Requirements Analysis

Tx board

- Wake up on 1st July at 8 AM (please make an easy accessible variable)

- Read data from GNSS (QUECTEL L86, feel free use tinyGPS library)

- Listen for 5 seconds, then sleep 25 seconds and keep repeating.

- If It’s 10 AM go to sleep

- If “Hello” is received, send GNSS data + ID

- Wait for shutoff signal for 30 seconds, send GNSS + ID every 5 seconds, then sleep (Whether shut off signal received or not)

- Wake up next day at 8 AM and repeat from step 2

Relay board MCU1

- Send “Hello”

- Wait for 8 seconds to receive

- If nothing received wait 2 seconds and repeat from step 1

- If message received send it via UART, then repeat from step1

Relay board MCU2

- Wait for UART message

- Send the message to base station

Base Station

- Wait for Lora message

- Send the message via Bluetooth (UART)

Requirements Analysis

Tx board

- Wake up on 1st July at 8 AM (please make an easy accessible variable)

- Read data from GNSS (QUECTEL L86, feel free use tinyGPS library)

- Listen for 5 seconds, then sleep 25 seconds and keep repeating.

- If It’s 10 AM go to sleep

- If “Hello” is received, send GNSS data + ID

- Wait for shutoff signal for 30 seconds, send GNSS + ID every 5 seconds, then sleep (Whether shut off signal received or not)

- Wake up next day at 8 AM and repeat from step 2

Relay board MCU1

- Send “Hello”

- Wait for 8 seconds to receive

- If nothing received wait 2 seconds and repeat from step 1

- If message received send it via UART, then repeat from step1

Relay board MCU2

- Wait for UART message

- Send the message to base station

Base Station

- Wait for Lora message

- Send the message via Bluetooth (UART)

Timeline

We will start by doing simple example for each module and make sure they are working smoothly, then start building the application step by step.

Step 0 Schematic and layout review

STEP 1: LoRa communication Relay board and TX board

Please create a code of communication between two MCUs using LoRa communication at 902 MHz minimum Bandwidth, make sure sending and receiving working smoothly (Like a ping pong example from ST32 example). Please ensure the TCXO is properly being powered (there will be two pins to power it).Can you debug two boards at the same time? How are you planning on powering the boards? What programmers do you have?

- Yes I have debugger but only one. In order to debug 2 x STM32 MCUs together, I can manage to switch between.
- I have bench suppliers available up to 3, I can use them to power the boards.

STEP 2: GNSS Module Quectel L86 in TX board

Please ensure you can read date and time correctly from GNSS module. The time zone will be provided and an easy formula can be developed to transform from UTC to local time. Feel free to use any parser such as TinyGPS

STEP 3: RTC in TX board

Ensure smooth working of RTC, Correct power consumption MCU 2 uA and GNSS module 6 uA [GNSS module can be left on sleep mode to retain previous location; this allows to get new data much faster (cold start vs hot start)]. Ensure that the timers are working properly and wake up on time. Make sure TCXO is not powered while sleeping. There is a working GPS parser if needed

STEP 4: Sleep mode (5 days)

I will make an example of the MCU sleeping and wake up at certain time using RTC and test this to ensure that it’s working accurately.

STEP 4: Relay board MCU1 Application and TX board interaction

Please write the code of the first MCU of the relay board. LORA communication tested on step 1. Please check the Hello signal and transmission of the GNSS + ID data Between TX board and Relay board.

STEP 5: Testing and validation the main part

Please test the application to ensure smooth operation with two Tx boards to ensure that increasing the number of boards doesn’t affect the performance. Please provide a verification for this. I can test with more TX boards in the USA and at long range.

STEP 6: Relay board MCU2 Application

Please test the application to ensure smooth operation with Base Station and relay

STEP 7: Base station Application

Please develop an iOS app to map the location of the TX bords. An image from Satellite can be used to locate the different TX board

STEP 8: Merge and testing

Testing the whole application with all the boards connected, then testing with the mobile App.(only takes into account testing with mobile app and not testing of the mobile app)